# PRACTICAL – 2

# AIM

To study and understand the different types of network cables used in computer networking.

# OBJECTIVE

To identify the different types of network cables, their categories, data transmission speeds, advantages, disadvantages, and applications.

# THEORY

Network cables are physical transmission media used to connect computers, networking devices, and other communication equipment. The commonly used network cables are:

- Unshielded Twisted Pair (UTP) Cable

- Shielded Twisted Pair (STP) Cable

- Shielded and Screened Twisted Pair (SSTP) Cable

- Coaxial Cable

- Fibre Optic Cable

## 1. Unshielded Twisted Pair (UTP) Cable

UTP cable consists of pairs of insulated copper wires twisted together without any external shielding. It is inexpensive, easy to install, and widely used in Ethernet networks.

Categories and Applications

| Category | Maximum Data Transmission | Application / Use |
| --- | --- | --- |
| Category 3 (Cat 3) | Up to 10 Mbps | 10Base-T Ethernet |
| Category 5 (Cat 5) | Up to 100 Mbps | Fast Ethernet |
| Category 5e (Cat 5e) | Up to 1 Gbps | Fast Ethernet, Gigabit Ethernet |

Advantages

- Cheaper in cost.

- Easy to install.

- Has a smaller overall diameter.

- Lightweight and flexible.

Disadvantages

- More prone to electromagnetic interference (EMI).

- More susceptible to external noise.

- Performance can be affected by interference over longer distances.

## 2. Shielded Twisted Pair (STP) Cable

STP cable is a twisted-pair cable with additional shielding around the wires. The shielding reduces electromagnetic interference and improves signal quality.

Categories and Applications

| Category | Maximum Data Transmission | Application / Use |
| --- | --- | --- |
| Category 6 (Cat 6) | Up to 10 Gbps* | Gigabit Ethernet, high-speed networks |
| Category 6a (Cat 6a) | Up to 10 Gbps | 10G Ethernet, data centres |

*The actual supported speed and distance depend on the network standard and installation conditions.

Advantages

- Provides shielding against electromagnetic interference.

- Less susceptible to noise and interference.

- Supports higher-speed networking.

- Provides better signal quality than unshielded cables in noisy environments.

Disadvantages

- More expensive than UTP.

- Requires greater installation effort.

- Proper grounding is required for effective shielding.

## 3. Shielded and Screened Twisted Pair (SSTP) Cable

SSTP cable provides additional protection against electromagnetic interference by using shielding around the individual twisted pairs and/or the complete cable.

Category and Application

| Category | Maximum Data Transmission | Application / Use |
| --- | --- | --- |
| Category 7 (Cat 7) | Up to 10 Gbps | Gigabit Ethernet, 10G Ethernet |

Advantages

- Excellent protection against electromagnetic interference.

- Supports high-speed data transmission.

- Suitable for environments with high electrical noise.

Disadvantages

- More expensive than UTP.

- More difficult to install.

- Requires careful termination and grounding.

## 4. Coaxial Cable

Coaxial cable consists of a central copper conductor surrounded by insulation, a metallic shield, and an outer protective jacket. It is commonly used for television networks and broadband Internet connections.

Common Types

| Type | Typical Application |
| --- | --- |
| RG-6 | Television and high-speed Internet connections |
| RG-59 | Television and video applications |
| RG-11 | Longer-distance cable and broadband applications |

Data Transmission

Coaxial cables can support data rates ranging from approximately 10 Mbps to 100 Mbps or higher depending on the cable type, network technology, and equipment used.

Advantages

- High bandwidth.

- Good resistance to electromagnetic interference.

- Low signal loss compared with some other copper cables.

- Versatile and widely used.

Disadvantages

- Limited transmission distance depending on the application.

- Can be relatively expensive.

- Bulky compared with twisted-pair cables.

## 5. Fibre Optic Cable

Fibre optic cable transmits data using pulses of light through optical fibres instead of electrical signals. It provides very high bandwidth and is suitable for long-distance communication.

Types of Fibre Optic Cable

- Single-Mode Fibre: Uses a small core and is suitable for long-distance, high-speed communication.

- Multi-Mode Fibre: Uses a larger core and is generally used for shorter-distance communication, such as within buildings and data centres.

Data Transmission

Fibre optic networks can support data rates of 100 Gbps and beyond, depending on the fibre type, network equipment, and transmission technology.

Advantages

- Very high transmission speed.

- Very high bandwidth.

- Suitable for long-distance communication.

- Highly resistant to electromagnetic interference.

- Provides high security and low signal loss.

Disadvantages

- More expensive than copper cables.

- Requires skilled installation and maintenance.

- Fibre is more delicate and requires careful handling.

# COMPARISON OF NETWORK CABLES

| Cable Type | Data Transmission | Main Advantages | Main Disadvantages | Applications |
| --- | --- | --- | --- | --- |
| UTP | Up to 1 Gbps for common Cat 5e installations | Low cost, easy installation | More susceptible to EMI | Ethernet networks |
| STP | Up to 10 Gbps | Shielded, less interference | Expensive, difficult installation | Gigabit and 10G Ethernet |
| SSTP | Up to 10 Gbps | Excellent interference protection | Expensive, requires careful installation | 10G Ethernet, high-noise environments |
| Coaxial | Typically 10–100 Mbps or higher depending on technology | Good shielding, versatile | Bulky, distance limitations | TV and broadband Internet |
| Fibre Optic | 100 Gbps and beyond | Very high speed, bandwidth and distance | Expensive, skilled installation required | Data centres, backbone networks, long-distance communication |

# RESULT

The different types of network cables—UTP, STP, SSTP, coaxial, and fibre optic—were studied. Their categories, transmission speeds, advantages, disadvantages, and common applications were identified and compared.