## Experiment 5: Filtering and Inspecting IP/ICMP Packets

## Aim

To capture 100 packets from the LAN interface using Wireshark, apply a display filter to show only IP/ICMP packets, and inspect the packet details.

## Procedure

- Open Wireshark and select the Local Area Connection interface.

- Go to Capture → Options.

- Tick “Stop capture automatically after 100 packets”.

- Click Start to begin the capture.

- Type ip || icmp in the display filter bar and press Enter.

- Select a packet and expand the Frame, Ethernet II, Internet Protocol and UDP/ICMP sections in the Packet Details pane to inspect it.

- Save the captured packets using File → Save.

## Display Filter Used

| Filter | ip ¦¦ icmp |
| --- | --- |
| Protocols displayed | IPv4 traffic including ICMP (NBNS, SSDP, LLMNR, DHCP, etc.) |

## Output

Figure 5.1: Wireshark output with the IP/ICMP display filter applied

## Observation

- Every packet shown has an IPv4 header containing the source and destination IP addresses.

- Packet 1 is a NetBIOS name query from 172.16.10.83 sent to the broadcast address 172.16.11.255 over UDP port 137.

- LLMNR queries are sent to the multicast address 224.0.0.252, and SSDP uses 239.255.255.250.

- The Packet Bytes pane shows the raw hexadecimal data of the selected frame.

## Result

The IP/ICMP packets were successfully filtered, captured and inspected using Wireshark.